Bill of material for the balancer

The components can be looked up in the Farnell online catalog (http://www.farnell.com) under the given order codes. Some of the components are common (such as the resistors), so no codes are given. Other components (as the inductors) are bought locally, so it will have to be figured out where they are available (now included in the basic PCB set !).

The prices are in DM and are for orientation only.

Bill of material for the trigger / motor controller / power supply assembly

| No | Name | Value | Package | Comment | Farnell # | Price |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | BR1 | B80C800 | Rect4 |  | 572-755 | 0,74 |
| 2 | C1 | 470µ | ELKO2 | Low ESR type 25V | 580-508 | 1,58 |
| 3 | C2 | 1n | C2 |  | 143-685 | 0,28 |
| 4 | C3 | 470µ | ELKO2 | Low ESR type 25V | 580-508 | 1,58 |
| 5 | C4 | 470µ | ELKO2 | Low ESR type 25V | 580-508 | 1,58 |
| 6 | C5 | 2,2mF | ELKO2 | 16V | 580-454 | 1,93 |
| 7 | C6 | 2,2mF | ELKO2 | 16V | 580-454 | 1,93 |
| 8 | C7 | 100n | C2 |  | 303-896 | 0,23 |
| 9 | C8 | 100n | C2 |  | 303-896 | 0,23 |
| 10 | C9 | 100n | C2 |  | 303-896 | 0,23 |
| 11 | C10 | 100n | C2 |  | 303-896 | 0,23 |
| 12 | C11 | 100n | C2 |  | 303-896 | 0,23 |
| 13 | C12 | 470µ | ELKO2 | Low ESR type 25V | 580-508 | 1,58 |
| 14 | C13 | 100n | C2 |  | 303-896 | 0,23 |
| 15 | C14 | 1n | C2 |  | 143-685 | 0,28 |
| 16 | C15 | 100n | C2 |  | 303-896 | 0,23 |
| 17 | C16 | 100n | C2 |  | 303-896 | 0,23 |
| 18 | C17 | 100n | C2 |  | 303-896 | 0,23 |
| 19 | C18 | 100n | C2 |  | 303-896 | 0,23 |
| 20 | C19 | 100n | C2 |  | 303-896 | 0,23 |
| 21 | D1 | LED | LED1ST | Colour according to preference | (656-471) | 0,36 |
| 22 | D2 | 1N5817 | D5 |  | 302-2742 | 0,46 |
| 23 | D3 | 1N4148 | D2 |  | 368-106 | 0,14 |
| 24 | D4 | 1N4148 | D2 |  | 368-106 | 0,14 |
| 25 | F1 | 150mAT | MYFUSE | 300mA for 115V configuration |  | 1,00 |
| 26 | GEH1 | P-EURO | P-EURO | PCB as supplied |  | 30,00 |
| 27 | GEH2 | Heatsink | Heatsink |  | 175-650 | 2,74 |
| 28 | GEH3 | Heatsink | Heatsink |  | 175-650 | 2,74 |
| 29 | IC1 | LMC662CN | DIL8 | Or equivalent | 401-778 | 4,79 |
| 30 | IC2 | 7908 | TO-220ST |  | 413-768 | 1,94 |
| 31 | IC3 | 7808 | TO-220ST |  | 412-995 | 1,68 |
| 32 | JP1 | Wire | 2X02 |  |  | 0,00 |
| 33 | L1 | 2mH | L3 |  |  | 1,50 |
| 34 | L2 | 2mH | L3 |  |  | 1,50 |
| 35 | R1 | 82R | R3 |  |  | 0,10 |
| 36 | R2 | 10k | R3 |  |  | 0,10 |
| 37 | R3 | 10k | R3 |  |  | 0,10 |
| 38 | R4 | 1k | R3 |  |  | 0,10 |
| 39 | R5 | 10k | R3 |  |  | 0,10 |
| 40 | R6 | 10k | R3 |  |  | 0,10 |
| 41 | R7 | 20k | 1X03 | 10 turn pot, connected by wire | 351-805 | 20,62 |
| 42 | R8 | 220R | R3 |  |  | 0,10 |
| 43 | R9 | 22k | R3 |  |  | 0,10 |
| 44 | R10 | 47k | R3 |  |  | 0,10 |
| 45 | R11 | 470R | R3 |  |  | 0,10 |
| 46 | R12 | 47k | 1X03 | Cermet pot, connected by wire | 614-221 | 9,59 |
| 47 | R13 | 10k | R3 |  |  | 0,10 |
| 48 | T1 | BD139 | TO-126ST | Or similar | 334-3315 | 1,65 |
| 49 | T2 | BUZ10 | TO-220ST | Or similar | 353-980 | 1,91 |
| 50 | TR1 | 2x115/2x9 | T-FLE12-24 | PCB resin molded transformer | 899-525 | 26,53 |
| 51 | X1 | D-SUB9F | DSUB9FL | Horizontal PCB female D-Sub9 | 150-754 | 4,83 |
| 52 | X2 | BNC | 1X02 | BNC jack, connected by wire | 583-522 | 3,98 |
| 53 | X3 | K1X2 | Clamp-2 | Mains wire connection clamp | 333-5938 | 1,02 |
| 54 |  |  |  | Fuse Clamps (2 Pcs.) | (146-123) | 0,49 |
| 55 |  |  |  | Dil 8 socket | 738-517 | 0,50 |
| 56 |  |  |  | M3*10 Screw (2 Pcs.) |  | 0,20 |
| 57 |  |  |  | M3 washer (2 Pcs.) |  | 0,10 |
| 58 |  |  |  | M3 nut (2 Pcs.) |  | 0,20 |
| 59 |  |  |  | LED white ultra-bright |  | 6,50 |
| 60 |  |  |  | Connector D-Sub9 male solder | 698-064 | 1,20 |
| 61 |  |  |  | Cover for D-Sub9 | 469-944 | 2,74 |
|  |  |  |  |  | Sum | 142,76 |

Bill of material for the piezo buffer. For the balancer two of the buffers are required.

| No | Name | Value | Package | Comment | Farnell # | Price |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | C1 | 1µ | 1206 | Ceramic multi-layer | 757-652 | 0,36 |
| 2 | C2 | 1µ | 1206 | Ceramic multi-layer | 757-652 | 0,36 |
| 3 | IC1 | LMC662CM | SO8 |  | 594-570 | 4,49 |
| 4 | JMP1 | 1206-JMP | 1206-JMP | Jumper | 772-240 | 0,08 |
| 5 | JMP2 | 1206-JMP | 1206-JMP | Jumper | 772-240 | 0,08 |
| 6 | X1 | K1X1 | 1X01 | Piezo connector - not supplied |  | 0,00 |
| 7 | X2 | K1X7 | 1X07-SMD |  | 341-8339 | 0,38 |
| 8 | R1 | 10k | 1206 |  |  | 0,10 |
| 9 | R2 | 10k | 1206 |  |  | 0,10 |
| 10 | R3 | 10k | 1206 |  |  | 0,10 |
| 11 | R4 | 10k | 1206 |  |  | 0,10 |
| 12 | R5 | 3.3M | 1206 |  |  | 0,10 |
| 13 |  |  |  | PCB as supplied |  | 5,00 |
| 14 |  |  |  | 7 pin connector to fit K2 | 341-9095 | 0,84 |
|  |  |  |  |  | Sum | 12,09 |